# Voltie Charger — Installation and Usage Guide

Step-by-step guide for installing the Voltie Charger Home Assistant integration and the companion dashboard card.

- Integration repo: https://github.com/voltie-eu/homeassistant-voltie_charger

- Card repo: https://github.com/voltie-eu/lovelace-voltie-charger-card

⚠️ Upgrading from v0.1.x to v0.2.0 is a breaking change. Remove the existing Voltie Charger integration before updating. Entity IDs and the underlying schema changed and are not automatically migrated.

## Contents

1. Prerequisites

2. Install the Integration

3. Connect the Integration to the Charger

4. Install the Dashboard Card

5. Add the Card to a Dashboard

6. Settings and Usage

## 1. Prerequisites

### 1.1 A Voltie Charger on the local network

The charger must be powered on, on the same LAN as the Home Assistant instance, and reachable by IP. Note the charger's IP — you may need it later.

### 1.2 Home Assistant (2025.1 or newer)

If you don't already have Home Assistant, follow the official guide:

- Home Assistant installation: https://www.home-assistant.io/installation/

Check the installed version under Settings → About.

### 1.3 HACS (Home Assistant Community Store)

HACS is a third-party package manager for Home Assistant. This integration and card are installed and updated through it.

If you don't already have HACS, follow the official guide:

- HACS installation: https://www.hacs.xyz/docs/use/download/download/

Once HACS is in the sidebar, you're ready.

### 1.4 HTTP API enabled on the Charger

The integration talks to the charger over its local HTTP API, which is disabled by default. Turn it on in the Voltie mobile app before starting the install.

If you set a username and password on the HTTP API, note them down — you'll need them in step 3.

## 2. Install the Integration

1. Open HACS in the sidebar.

2. Open the ⋮ menu → Custom repositories.

3. Fill in the dialog:

  - Repository: https://github.com/voltie-eu/homeassistant-voltie_charger

  - Type: Integration

4. Click Add.

5. Back on the HACS main page, search for Voltie Charger.

6. Click the result and click Download.

7. Confirm the latest version and click Download again.

8. Restart Homeassistant

## 3. Connect the Integration to the Charger

After the integration is installed, link it to the specific charger on the network.

### 3.1 Automatic discovery (most common)

Home Assistant finds the charger via mDNS within about a minute of startup.

1. Go to Settings → Devices & services.

2. A Voltie Charger appears with the charger's ID under the Discovered.

3. Click Add.

4. If the charger has credentials set, enter the Username and Password. Otherwise the form submits directly.

5. Click Submit, then Finish on the success dialog.

### 3.2 Manual setup (when discovery fails)

Use this if no Discovered notification appears after a minute. Typical on VLAN-isolated networks where mDNS is blocked between segments.

1. Go to Settings → Devices & services → Add integration.

2. Search for Voltie Charger and click it.

3. Fill in the form:

  - Host (required): the charger's IP address, e.g. 192.168.1.50. No http:// prefix, no port. Find the IP in your router's DHCP client list or in the Voltie app under Settings → Network.

  - Username (optional): the HTTP API username. Leave empty if no credentials were set.

  - Password (optional): the HTTP API password. Leave empty if no credentials were set.

4. Click Submit, then Finish.

## 4. Install the Dashboard Card

Install the card after the integration is working.

1. Open HACS.

2. ⋮ → Custom repositories.

  - Repository: https://github.com/voltie-eu/lovelace-voltie-charger-card

  - Type: Dashboard

3. Click Add.

4. Search for Voltie Charger Card, click the result, click Download.

5. Confirm the latest version and click Download again.

6. Hard-refresh the browser (⌘/Ctrl + Shift + R).

No Home Assistant restart is needed for cards.

## 5. Add the Card to a Dashboard

1. Open the dashboard where you want the card. (If you don't have one yet, see the official Home Assistant documentation on how to create one.)

2. Click Edit dashboard in the top right corner.

3. Select where you want to add the card.

4. Search for Voltie and pick Voltie Charger.

5. Select your charger in the Voltie Charger device dropdown.

6. Click Save, then Done to exit edit mode.

## 6. Settings and Usage

### 6.1 Changing the charger's IP address or credentials

If the charger's IP changes, or you update the HTTP API username/password in the Voltie app, update the integration:

1. Go to Settings → Devices & services → Voltie Charger.

2. Click the ⋮ menu on the entry → Reconfigure.

3. Update Host, Username, or Password as needed.

4. Click Submit.

Home Assistant also prompts automatically with a "Repair" notification when it detects that credentials have stopped working.

### 6.2 Adjusting the polling interval

The integration polls the charger every 30 seconds by default. To change this:

1. Go to Settings → Devices & services → Voltie Charger.

2. Click Configure (gear icon) on the entry.

3. Set Scan interval in seconds. Allowed range: 5 to 300.

4. Click Submit.

Lower values give a more responsive UI and a smoother live power graph in the card; higher values reduce LAN traffic.

### 6.3 Charger controls

The integration exposes the charger as a regular Home Assistant device. Open it under Settings → Devices & services → Voltie Charger → click the device.

- Start / stop charging: toggle the Charging switch (or use the slider on the dashboard card).

- Maximum charging current: enter the desired Maximum charging current, value in amps.

- Autostart, Display, LEDs, Buzzer: toggle the corresponding switches.

Because these are standard entities, you can also use them in automations and scripts.

### 6.4 Downloading diagnostics

For support tickets, attach a redacted diagnostics dump:

1. Settings → Devices & services → Voltie Charger.

2. Click ⋮ on the entry → Download diagnostics.

3. Attach the resulting .json file. Credentials are stripped before download.

### 6.5 Removing the integration

1. Settings → Devices & services → Voltie Charger → click ⋮ → Delete.

2. Confirm.

This removes all entities and the device. The dashboard card will show "Choose a charger" until the integration is re-added.